# FUN101 – Week 7 Day 2 Administrative Lesson Plan

Chapter 6 – Vehicle Hoists, Cranes, Rigging Equipment, and Wire Rope Inspection

## Configuration Board

| Warm Up | Students examine photos of a vehicle hoist, an engine crane, and a wire rope sling and identify the lifting task each is designed for. |
| --- | --- |
| Objective | Identify vehicle hoist types and certification requirements, describe crane and rigging equipment use, and inspect wire rope for safe operating condition. |
| Standard | CDX Chapter 6 learning objectives and shop safety outcomes. |
| Essential Question | Why does the safe working load of a rigging assembly equal its weakest component's rating, not the average? |
| Exit | Students identify a wire rope's SWL tag information and demonstrate the glove-and-towel wire rope inspection method. |

## Objectives

Students will identify vehicle hoist types and their certification requirements, describe the types and uses of cranes and rigging equipment, and explain how to inspect wire rope for safe operating condition.

## Necessary Materials

CDX Chapter 6 presentation, shop vehicle hoist, engine crane or shop crane, rigging equipment samples (sling, shackle, chain), wire rope sample, inspection checklist.

## Academic Activities

- Compare vehicle hoist types (platform, single post, two post, scissor lift, portable) and their locking mechanisms and rated capacities.

- Review OSHA inspection and certification requirements for lifting and hoisting equipment.

- Discuss crane types (mobile, gantry, engine, overhead) and rigging equipment (chain blocks, ropes, chains, cables, slings, shackles, eye hooks).

- Explain safe working load (SWL), why the SWL of an assembly equals its weakest component, and how sling angle affects lifting capacity.

- Introduce wire rope construction, designation, end terminations, and the manufacturer inspection requirements for load capacity and wear limits.

## Shop Activities

- Inspect a shop vehicle hoist's locking mechanism and rated-capacity plate.

- Identify rigging equipment samples and their SWL tags.

- Calculate the effective lifting capacity of a sling at a given angle from vertical.

- Attach a load to a crane at the correct lift point relative to the load's center of gravity.

- Inspect a wire rope sample for broken wires, kinks, birdcaging, and diameter reduction using the glove-and-towel method.

## Industry Applications

Vehicle hoists, cranes, rigging equipment, and wire rope inspection apply across automotive and diesel shops, heavy equipment yards, agricultural service centers, and marine repair facilities wherever equipment must be lifted safely from above.

## Common Technician Errors

- Exceeding a hoist's rated capacity because the vehicle “looks light enough.”

- Attaching a lifting point that isn't vertically in line with the load's center of gravity.

- Mixing rigging components without checking that each one is rated for the job.

- Returning a wire rope to service without checking for broken wires or diameter loss.

## Captain Joe's Shop Tip

The SWL tag isn't a target — it's a ceiling. When in doubt, rig for the job you're doing, not the job you wish was smaller.

## Assessment

Instructor observation, guided questioning, participation, practical demonstration, and exit activity.

## Homework

Complete Chapter 6 review questions 1–4, 6, and 7, and review the shop's hoist and rigging equipment capacity ratings.

## Instructor Reflection

Student strengths:

Student challenges:

Concepts requiring additional review: