# DSL204 – Week 5 Day 1 Administrative Lesson Plan

Chapter 37 – ABS Fundamentals, Requirements, Components, and Configurations (CDX LO 37-1 – 37-4)

Time: 4 Hrs (2 Hrs Classroom / 2 Hrs Shop)

## Configuration Board

| Warm Up | Students discuss a time they felt a vehicle's brakes pulse or grab during hard braking and relate the sensation to wheel lock-up. |
| --- | --- |
| Objective | Explain how an antilock braking system (ABS) prevents wheel lock-up, describe FMVSS 121 system requirements, and identify basic ABS components and configurations. |
| Standard | CDX Chapter 37 learning objectives 37-1 through 37-4 and shop safety outcomes. |
| Essential Question | Why does preventing wheel lock-up improve both stopping distance and steering control at the same time? |
| Exit | Students identify ABS components on a tractor/trailer diagram and state the sensor/modulator configuration for a given axle count. |

## Student Objectives

Students will describe ABS fundamentals and their stopping-distance and steering-control benefits, identify FMVSS 121 system requirements, describe ABS components (ECU, wheel speed sensors, modulator valves) and their functions, and identify common tractor and trailer ABS configurations.

## Necessary Materials

CDX Chapter 37 presentation, shop tractor/trailer with ABS, ABS wiring diagrams, wheel speed sensor and modulator valve samples or shop units, diagnostic scan tool, multimeter, manufacturer service manual, inspection checklist.

## Terminology

ABS, FMVSS 121, electronic control unit (ECU), wheel speed sensor, modulator valve, exhaust / hold / reapply, Chuff Test, 4S/4M, 6S/4M, 6S/6M, power line carrier.

## Academic Activities

- Discuss how ABS electronically measures individual wheel speeds and modulates brake pressure to stay just below the wheel lock-up threshold.

- Review FMVSS 121 stopping-distance and stability requirements and how they apply to tractors, trailers, and converter dollies.

- Identify ABS components and their functions: the ECU, wheel speed sensors, modulator valves, and the trailer ABS power and warning-lamp circuit.

- Explain the three phases of the antilock braking cycle – exhaust, hold, and reapply – and the purpose of the Chuff Test.

- Compare common tractor configurations (4S/4M, 6S/4M, 6S/6M) and trailer configurations (2S/1M through 4S/3M).

## Shop Activities

- Locate and identify ABS components (ECU, wheel speed sensors, modulator valves) on a shop tractor and trailer.

- Trace the trailer ABS power and warning-lamp circuit from the stop-lamp connector to the ABS module.

- Perform a Chuff Test to validate modulator valve operation.

- Identify the ABS configuration (sensors/modulators) on the shop vehicle and compare it to the service manual.

- Retrieve and record ABS fault information using a diagnostic scan tool or blink codes.

## Industry Applications

ABS is standard on tractors, trailers, and converter dollies, is common on motor coaches, and is increasingly used on agricultural and construction equipment operating on public roads; the same wheel-speed-sensor and modulator-valve principles apply across air and hydraulic brake systems.

## Common Technician Errors

- Assuming a trailer with no power to the ABS module has a bad ECU rather than checking the stop-lamp circuit first.

- Ignoring a sensor air-gap check after wheel-end service.

- Misreading the ABS configuration and ordering the wrong replacement modulator or sensor.

- Skipping the Chuff Test and assuming a modulator is good without functional verification.

## Instructor's Shop Tip

A lit ABS lamp doesn't always mean a bad ABS. Check the simple things first — connector, ground, and stop-lamp power — before you condemn the module.

## Assessment

Instructor observation, guided questioning during component identification, participation in the shop rotation, practical demonstration of the Chuff Test and fault-code retrieval, and exit activity.

## Homework

Complete Chapter 37 review questions 1–5 and review the ABS configuration and wiring diagram for the shop's demonstration tractor/trailer.

## Notes / Flex Activities

If time allows, incorporate the “You Are the Technician” case study discussion from the chapter opener. Early finishers may trace the ABS circuit on a second unit.

## Instructor Reflection

Student strengths:

Student challenges:

Concepts requiring additional review: