# FUN101 – Week 7 Day 1 Administrative Lesson Plan

Chapter 6 – Manual Lifting, Jacks, Stands, and Chocks

## Configuration Board

| Warm Up | Students discuss why lifting with the legs instead of the back matters and identify the weight limit for a safe manual lift. |
| --- | --- |
| Objective | Demonstrate proper manual lifting technique and safely use jacks, jack stands, and wheel chocks to lift and support a vehicle. |
| Standard | CDX Chapter 6 learning objectives and shop safety outcomes. |
| Essential Question | Why is a jack alone never enough to support a vehicle you're going to work under? |
| Exit | Students demonstrate correct manual lifting posture and the jack-and-stand support sequence. |

## Objectives

Students will demonstrate safe manual lifting technique, identify lifting hazards, and safely use jacks, jack stands, and wheel chocks to lift and support a vehicle for service.

## Necessary Materials

CDX Chapter 6 presentation, floor jack, bottle jack, matched pair of jack stands, wheel chocks, shop vehicle, PPE, inspection checklist.

## Academic Activities

- Review OSHA manual lifting guidelines and the NIOSH 51 lb (23 kg) single-person lifting limit.

- Discuss the types of jacks (hydraulic, pneumatic, mechanical) and their applications (floor, bottle, sliding-bridge, transmission, clutch).

- Explain why a jack is used only to raise and lower a vehicle and never to support it during service.

- Review correct wheel-chock placement and the parking-brake and neutral procedure before lifting.

- Demonstrate the jack-and-jack-stand sequence: lift, position stands, lower onto stands, and the reverse sequence for lowering.

## Shop Activities

- Practice proper manual lifting posture using a shop-safe load.

- Identify jack types available in the shop and match each to its correct application.

- Chock the wheels and set the parking brake on a shop vehicle before lifting.

- Raise a vehicle with a floor jack and support it on a matched pair of jack stands at the correct lift points.

- Lower a vehicle from jack stands using the correct reverse sequence.

## Industry Applications

These lifting and support procedures apply to passenger vehicles, diesel trucks, RVs, trailers, heavy equipment, agricultural equipment, and marine service, anywhere a vehicle must be safely raised and supported for underbody work.

## Common Technician Errors

- Lifting a heavy load with the back instead of the legs.

- Getting under a vehicle supported only by a jack.

- Using mismatched or incorrectly rated jack stands.

- Failing to chock the wheels before lifting.

## Captain Joe's Shop Tip

A jack lifts. A jack stand holds. Never confuse the two jobs, and never trust your life to just one of them.

## Assessment

Instructor observation, guided questioning, participation, practical demonstration, and exit activity.

## Homework

Complete Chapter 6 review questions 5, 8, and 9, and review the shop's jack and jack stand capacity ratings.

## Instructor Reflection

Student strengths:

Student challenges:

Concepts requiring additional review: